OPTIONS for high-speed roll-up door, type “EFA-SRT®-ECO”

Lining of the winding shaft/motor:

Motor covering

complete covering over motor and shaft

Surface:

Powder coating of all galvanised steel parts in a colour according to RAL __________

(pearl, luminous and metallic colours not possible)

or

stainless steel version (V2A) of all visible steel parts, visible surface polished, coarseness 220,

incl. switch cabinet made of V2A, e.g. for steaming

Crash version:

Surcharge for special displacement mechanism at the bottom section in order to avoid unintentional starting-up. In the case of an unintentional collision, the bottom end shield can be pushed out of the guides on both sides. Switches which are arranged on both sides will then report the “crash” to the control so that the door operation is stopped immediately. After manually reguiding the bottom end shield again, and after acknowledging at the switch cabinet, automatic operation can be restarted. With this door variant the signals of the crash switches and the contact edge are sent to the control via remote impulse. (A spiral shape cable connected to the door blade is not wanted)

If necessary:

Automatic interim stop at a height of H = ___________ mm

Special curtains:

polyester fibre 2.0 mm thick, transversely stable, FDA approved

red / orange / yellow: non-antistatic, silicone free

blue / grey-white: antistatic, silicone-free,

as above, but

with transparent window made of fully transparent PVC

(in the case of door curtain with window only partly antistatic and partly

approved for the food industry)

Option:

polyester fibre 2.0 mm thick, black-grey, transversely stable, urethane-impregnated,

fire resistance class B1 (SE) according to DIN EN 20340 antistatic, silicone-free