OPTIONS for high-speed roll-up door, type „EFA-SRT-L/PREMIUM®“:

Cover of the winding shaft/motor:

Motor covering

front covering over motor and shaft

complete covering over motor and shaft

Surface:

Powder coating of all galvanised steel parts in a colour according to RAL __________

(pearl, luminous and metallic colours not possible)

or

stainless steel version (V2A) of all visible steel parts, visible surface polished, coarseness 220,

incl. switch cabinet made of V2A, e.g. for wet operations

Crash version:

Surcharge for special displacement mechanism on both sides at the bottom section which is used in order to avoid damages due to unintentional starting-up. In the case of an unintentional collision, the end shield can be pushed out of the guides on both sides. Switches which are arranged on both sides will then report the “crash” to the control so that the door operation is stopped immediately. After manually reguiding the end shield, and after acknowledging at the switch cabinet, automatic operation can be restarted.

If necessary:

Automatic interim stop at a height of H = ___________ mm

Special curtains:

polyester fibre 2.0 mm thick, transversely stable, FDA approved

red / orange / yellow: non-antistatic, silicone free

blue / grey-white: antistatic, silicone-free,

as above, but

with transparent window made of fully transparent PVC

(in the case of door curtain with window only partly antistatic and partly

approved for the food industry)

Option:

polyester fibre 2.0 mm thick, black-grey, transversely stable, urethane-impregnated,

fire resistance class B1 (SE) according to DIN EN 20340 antistatic, silicone-free