OPTIONS for high-speed roll-up door, type “EFA-SRT®-MTL”

Equipment package for “access restriction” (with personnel traffic):

Surcharge for application in areas with persons in the door area, consisting of:

Aluminium end-shield (E6-EV1) with integrated safety edge in rubber profile. Covered guidance of energy chain to the bottom edge with additional cover on one side of the door frame.

High-frequency drive with gear protection and absolute encoder, emergency opening via crank, microprocessor control in plastic box (IP 65) with main switch

Motor and winding shaft covering:

Complete covering over motor and shaft

Motor position:

Surcharge for space-saving, motor position, drive connection via chain

Surface:

Powder coating of all galvanised steel parts and aluminium parts in a colour according to RAL __________ (pearl, luminous and metallic colours not possible)

Frame extension:

Surcharge for extension of frames on both sides, e.g. for conveyor application at a higher level than floor level (infill between floor and top edge of conveyor to be supplied locally)

Surcharge for conveyor kit:

Separate terminals for the following inputs and outputs:

Inputs:		door open / door closed / emergency stop

Outputs:	door opened / door closed / door ready-failure

(max. with 24V DC)

Message of positions via mechanical limit switches:

Surcharge for mechanical limit switch, roller lever type for additional message door open / door closed to local facility

Message of positions via safety limit switches:

Surcharge for inductive safety limit switch, (category IV) for additional message door open / door closed to local facility. Evaluator for integration in local control supplied loosely.

Door curtain extension on one side:

Surcharge for extension of door curtain on one side (left hand side or right hand side selectable, min. height 350mm / max. height 1200mm, min. width 250mm), e.g. for laterally placed conveyor system.

Rubber lip extension:

Surcharge for extension of bottom rubber sealing lip (max. 300mm) for adaption to protruding parts of the conveyor.

Special curtains:

polyester fibre 2.0 mm thick, transversely stable, FDA approved

red / orange / yellow: non-antistatic, silicone free

blue / grey-white: antistatic, silicone-free,

without vision window

polyester fibre 2.0 mm thick, transversely stable, FDA approved

red / orange / yellow: non-antistatic, silicone free

blue / grey-white: antistatic, silicone-free,

with vision window

(in the case of door curtain with window only partly antistatic and partly

approved for the food industry)

polyester fibre 2.0 mm thick, black-grey, transversely stable, urethane-impregnated,

hardly flammable class B1 (SE) according to DIN EN 20340 antistatic, silicone-free

without vision window

polyester fibre 2.0 mm thick, black-grey, transversely stable, urethane-impregnated,

hardly flammable class B1 (SE) according to DIN EN 20340 antistatic, silicone-free

with translucent UV protection window acc. to DIN 1598